spectHR vs CARSPAN / VU-AMS

Algorithm Differences

spectHR is a faithful Python reimplementation of the CARSPAN cardiovascular analysis system. The vast majority of its algorithms are verified bit-for-bit ports of the CARSPAN Pascal source (T_AnaFunctions.pas). This document records the places where spectHR departs from or extends the originals, and flags where numbers will differ from CARSPAN or VU-DAMS output even when the method is mathematically equivalent.

# 1. Frequency Bands

Reference: CARSPAN manual §3.1.5; Task Force (1996).

spectHR uses the CARSPAN / Groningen frequency bands, which differ from the Task Force (1996) clinical standard:

| Band | spectHR (CARSPAN) | Task Force (1996) |
| --- | --- | --- |
| VLF | 0.02–0.06 Hz | < 0.04 Hz |
| LF | 0.07–0.14 Hz | 0.04–0.15 Hz |
| HF | 0.15–0.40 Hz | 0.15–0.40 Hz |
| FullRange | 0.02–0.50 Hz | 0.02–0.40 Hz |

The CARSPAN LF band is narrower and shifted upward, deliberately centred on the ~0.10 Hz Mayer-wave / baroreceptor oscillation that is most sensitive to mental effort (Mulder, 1980, 1989). spectHR’s LF and VLF values are NOT numerically interchangeable with Task Force LF/VLF. Band edges are user-configurable in the workspace.

# 2. PSD Methods

## 2.1 Default: CARSPAN IBI-amplitude DFT (Eq. 3.21)

Reference: CARSPAN manual Eq. 3.21; T_AnaFunctions.pas RunPDS.

Faithful port. The DFT is computed on the actual cumulative-IBI time grid — no uniform resampling. Each event is weighted by its IBI duration in seconds: Amp[i] = (IBI_ms[i] − mean_ms) × taper[i] × IBI_s[i]. Default for Profiles and Transfer views.

## 2.2 CARSPAN Event-Series DFT (Eq. 3.19)

Reference: CARSPAN manual Eq. 3.19; T_AnaFunctions.pas SOC branch.

R-peaks modelled as unit impulses. Faithful port; reproduces CARSPAN manual worked example to within ~2 % on every band.

## 2.3 Welch PSD — additional method

Divergence: CARSPAN does not use Welch. This is an added method.

IBI series resampled to 4 Hz by cubic interpolation, then scipy.signal.welch (Hann window, 256-sample segments, 50 % overlap). Returns ms²/Hz, converted to mMI²/Hz. Known limitation: interpolation distorts the spectrum at low beat counts; the CARSPAN method is therefore the default.

## 2.4 Lomb–Scargle PSD — additional method

Divergence: Not present in CARSPAN.

scipy.signal.lombscargle on mean-subtracted IBI values at native event times. Power: (2·T/N)·pgram. DOF estimation is approximate. Available as an opt-in alternative.

# 3. Normalisation

Reference: CARSPAN manual Eq. 3.20.

All methods offer both ms²/Hz and mMI²/Hz (squared modulation index). Conversion: 10⁶ / mean_IBI². Divergence — two mean-IBI conventions:

Harmonic mean (default): mean_IBI = T / N (segment length ÷ beat count).

Arithmetic mean (CARSPAN-strict): mean_IBI = 1 / mean(1/IBI_i) — used by the CARSPAN Pascal source.

The two conventions give slightly different normalised power values. The distinction matters when comparing absolute mMI² values between spectHR and original CARSPAN output.

# 4. Taper (Cosine-Bell Window)

Reference: T_AnaFunctions.pas:128–161 (Taper function).

spectHR’s cosine-bell taper is a bit-for-bit port of the Pascal original. It uses an (i+1) index offset, meaning the first and last samples are NOT zeroed — this differs from scipy’s Tukey window, which zeros boundary samples. Default: 5 % per side (Pascal TaperPercent = 5). Using scipy’s built-in windows with the CARSPAN engine will produce different spectral results.

# 5. Resampling to Display Grid

Reference: T_AnaFunctions.pas:171–239 (Resample_R).

For display, the native DFT grid (1/T spacing) is resampled to 0.01 Hz resolution using energy-conserving bin averaging with fractional-coverage weights. The upper boundary of each display bin is exclusive. Faithful Pascal port.

Important: this resampling applies to the displayed spectrum only. Band-power integrals use the native grid (CARSPAN manual §3.3.5), so plotted and exported numbers may appear to differ slightly.

# 6. Smoothing

Reference: T_AnaFunctions.pas:595–643 (MAW_R); lines 443–487 (CrossSpectrum WindowSize=3).

3-point MAW (display smoother): Applied to the plotted PSD curve when “Smooth for display” is enabled. Edge weights: out[0] = (3/8)·x[0] + (5/8)·x[1]; out[−1] = (5/8)·x[−2] + (3/8)·x[−1]. Faithful Pascal port. NOT applied to band-power integrals.

3-point triangular smoother (transfer / profile): Applied to auto- and cross-spectra in the profile and transfer pipelines (Pascal WindowSize=3). Yields sub-unity coherence estimates for sliding-window analysis. Edge policy: left mirror, right replicate-centre — faithful to Pascal.

# 7. RSA (Respiratory Sinus Arrhythmia)

Reference: Grossman et al. (1990); VU-DAMS manual §4.2.

Grossman peak-to-valley method: shortest IBI on the accelerating slope of each inhalation, longest IBI on the decelerating slope of each exhalation; RSA per breath = longest − shortest (ms). Default lag: 1.0 s (VU-DAMS default; use 0.3–0.5 s for children).

VU-DAMS rejection codes implemented: Code –5 rejects an IBI if deviation from the previous IBI exceeds 50 % (max_ibi_deviation); Code –6 rejects a whole breath if respiration rate deviates more than 50 % from the 20-breath running average (max_rate_deviation).

Two output metrics: rsa — mean of valid (positive) RSA values only; invalid breaths are NaN (Grossman convention). rsa0 — invalid breaths counted as zero in the denominator (VU-DAMS RSA0 convention). A researcher using only ‘rsa’ will obtain slightly higher means than VU-DAMS’s RSA0 when rejection rates are high.

# 8. Transfer Function / Spectral Baroreflex Sensitivity

Reference: T_AnaFunctions.pas RunTransfer (lines 2493–2610); CARSPAN manual §3.1.5.

Faithful port of the CARSPAN RunTransfer pipeline: H(f) = CrossSpectrum(input, IBI) / AutoSpectrum(input). Output channel is always the IBI series. Input is selectable: respiration (classic RSA transfer) or per-beat systolic/diastolic BP (spectral BRS, result in ms/mmHg).

Single-epoch coherence: Squared coherence is 1 at every non-zero bin by construction (one DFT realisation per epoch). The 3-point triangular smoother (smooth=True, the default for profile analysis) is required to obtain sub-unity estimates — matching CARSPAN’s WindowSize=3 profile branch.

Important caveat: Transfer-function BRS is an open-loop estimate of a closed-loop system (blood pressure and heart rate mutually drive each other). This is inherent to the method, not a bug.

# 9. Stationarity Metric

Reference: Bendat & Piersol (1971) reverse-arrangements test.

spectHR computes two stationarity measures: Pearson correlation of IBI vs. beat index (linear trend detector) and a reverse-arrangements z-score (counts pairs i < j with IBI[j] > IBI[i]).

Limitation: neither test detects variance non-stationarity or periodic trends. Wide-sense stationarity — required for valid PSD interpretation — is not fully verifiable with these metrics alone. See docs/ALGORITHM_AUDIT.md §1.6.

# 10. Frequency-Band Integration

Reference: CARSPAN manual Eq. 3.28.

Band power is computed by rectangular integration over the native frequency grid. The bin width for non-uniform grids uses centred differences for interior bins and one-sided differences for edge bins, correctly handling both the uniform Welch/Lomb–Scargle grids and the non-uniform CARSPAN native grid. Faithful to the Pascal integration logic.

# Summary

The table below lists every area reviewed. Entries in the Difference column that are not “None (faithful port)” or “None (bit-for-bit)” are shown in bold.

| Feature | CARSPAN Original | spectHR | Difference |
| --- | --- | --- | --- |
| PSD default | IBI-amplitude DFT | Same | None (faithful port) |
| Additional PSD | None | Welch, Lomb–Scargle | Added methods |
| Frequency bands | CARSPAN (LF 0.07–0.14) | Same | User-configurable |
| Taper | Cosine-bell (i+1 offset) | Same | None (bit-for-bit) |
| Display resampling | Resample_R | Same | None (bit-for-bit) |
| Smoothing | 3-pt MAW / triangular | Same | None (faithful port) |
| mMI² mean | Arithmetic | Harmonic (default) | Small numerical difference |
| RSA | Grossman peak-valley | Same + VU-DAMS guards | Guards optional; rsa vs rsa0 |
| Transfer | RunTransfer port | Same | None (faithful port) |
| Stationarity | Not in CARSPAN | Pearson r + reverse-arr. | Added; limited |
| Welch/LS normalisation | N/A | 10⁶/mean_IBI² | Added |

# References

Grossman, P., et al. (1990). A comparison of three quantification methods for estimation of respiratory sinus arrhythmia. Psychophysiology, 27, 702–714.

Mulder, G. (1980). The heart of mental effort. PhD thesis, University of Groningen.

Mulder, L. J. M. (1989). CARSPAN: cardiovascular spectral analysis.

Robbe, H. W. J., et al. (1987). Assessment of baroreceptor reflex sensitivity by spectral analysis. Hypertension, 10, 538–543.

Task Force of the ESC and NASPE (1996). Heart rate variability: standards of measurement. Circulation, 93, 1043–1065.

VU-DAMS manual V2 (2022). VU Ambulatory Monitoring System.